Faculty of Engineering, Architecture and Science

Department of Mechanical, Industrial, and Mechatronics Engineering

| Program | Electrical Engineering |
| --- | --- |
| Course Number | ELE754 |
| Course Title | Power Electronics |
| Semester/Year | F2025 |
| Instructor | Dr. David Xu |

| Lab/Tutorial Report NO. | Tutorial (1) |
| --- | --- |

| Report Title | Lab-Volt Power Converter and Data Acquisition Platform |
| --- | --- |

| Section No. | 02 |
| --- | --- |
| Submission Date | September 27, 2025 |
| Due Date | September 28, 2025 |

| Name | Student ID | Signature* |
| --- | --- | --- |
| Shai Mourad | 501170415 |  |

*By signing above you attest that you have contributed to this submission and confirm that all

work you have contributed to this submission is your own work. Any suspicion of copying or

plagiarism in this work will result in an investigation of Academic Misconduct and may result

in a “0” on the work, an “F” in the course, or possibly more severe penalties, as well as a

Disciplinary Notice on your academic record under the Student Code of Academic Conduct,

which can be found online at:

https://www.torontomu.ca/senate/policies/pol60.pdf.

## Table of Contents

Introduction	2

Objectives	2

Procedure	2

Output (Waveforms)	2

Data	7

Discussion	8

Conclusion	9

References	9

## Introduction

The tutorial laboratory introduces a Lab-Volt electric power technology platform, data acquisition software (LVDAC-EMS), and measurement instrumentation (digital multimeter and oscilloscope). Focusing on AC and DC signals the experiment focuses on different load and converter measurements, observing the relationship between input and output (based on knob position or duty cycle). This lab’s focus is to compare the results of both the software and hardware-based measurements and compare them for accuracy and consistency.

## Objectives

The objective of this tutorial is to become familiar with Lab-Volt Power Converters and Data Acquisition Platforms. Measuring AC voltage and current using LVDAC-EMS software with both a digital multimeter (DMM) as well as an oscilloscope is conducted using the aforementioned technology. Also, the lab studies a PWM buck chopper and records voltage/current at different duty cycles. All values/observations obtained are compared from software and hardware instruments.

## Procedure

Experiment 1:

A variable AC supply is connected to a 120 Ω resistor and 50 mH inductor. Measurements of voltage and current were taken via LVDAC-EMS software at knob positions 25%, 50%, 75%, and 100%..

Experiment 2:

Using the same circuit as “Experiment 1”, measurements are taken using a DMM (voltage) and oscilloscope (current). Waveforms are captured using the oscilloscope.

Experiment 3:

A PWM buck converter is connected to a 120 Ω resistor and 2 mH inductor. Measurements were taken via LVDAC-EMS + oscilloscope software at duty cycles 10%, 30%, 50%, 70%, and 90%.

## Output (Waveforms)

### Experiment 1 – LVDAC-EMS (AC Measurements)

Figure 1.1 – Voltage and Current Waveforms at 25% Knob | Figure 1.2 – Voltage and Current Waveforms at 50% Knob

Figure 1.3 – Voltage and Current Waveforms at 75% Knob | Figure 1.4 – Voltage and Current Waveforms at 100% Knob

### Experiment 2 – DMM & Oscilloscope (AC Measurements)

Figure 2.1 – Voltage and Current Waveforms at 25% Knob | Figure 2.2 – Voltage and Current Waveforms at 50% Knob

Figure 2.3 – Voltage and Current Waveforms at 75% Knob | Figure 2.4 – Voltage and Current Waveforms at 100% Knob

### Experiment 3 – PWM Buck Chopper (LVDAC-EMS)

Figure 3.1 – Voltage and Current Waveforms at 10% Duty Cycle | Figure 3.2 – Voltage and Current Waveforms at 30% Duty Cycle

Figure 3.3 – Voltage and Current Waveforms at 50% Duty Cycle | Figure 3.4 – Voltage and Current Waveforms at 70% Duty Cycle

Figure 3.5 – Voltage and Current Waveforms at 90% Duty Cycle

### Experiment 3 – PWM Buck Chopper (DMM & Oscilloscope)

Figure 3.6 – Voltage and Current Waveforms at 10% Duty Cycle | Figure 3.7 – Voltage and Current Waveforms at 30% Duty Cycle

Figure 3.8 – Voltage and Current Waveforms at 50% Duty Cycle | Figure 3.9 – Voltage and Current Waveforms at 70% Duty Cycle

Figure 3.10 – Voltage and Current Waveforms at 90% Duty Cycle

## Data

### Experiment 1 – LVDAC-EMS			Experiment 2 – DMM & Oscilloscope

| Knob | Voltage (V) | Current (A) |
| --- | --- | --- |
| 25% | 30.6 | 0.253 |
| 50% | 62.5 | 0.480 |
| 75% | 93.7 | 0.745 |
| 100% | 129.0 | 1.01 |

| Knob | Voltage (V) | Current (A) |
| --- | --- | --- |
| 25% | 27.17 | 0.215 |
| 50% | 57.65 | 0.453 |
| 75% | 86.90 | 0.687 |
| 100% | 118.9 | 0.940 |

### Experiment 3 – LVDAC-EMS			  Experiment 3 – DMM & Oscilloscope

| Duty Cycle | Voltage (V) | Current (A) |
| --- | --- | --- |
| 10% | 11.2 | 0.022 |
| 30% | 34.5 | 0.0255 |
| 50% | 57.12 | 0.059 |
| 70% | 79.15 | 0.065 |
| 90% | 100.4 | 0.051 |

| Duty Cycle | Voltage (V) | Current (A) |
| --- | --- | --- |
| 10% | 11.76 | 0.121 |
| 30% | 33.25 | 0.331 |
| 50% | 56.51 | 0.520 |
| 70% | 82.61 | 0.692 |
| 90% | 106.8 | 0.859 |

## Discussion

Both “Experiment 1 & 2” show near-linear increase in voltage and current with knob position. DMM/oscilloscope readings were consistently slightly higher than LVDAC-EMS values (e.g., at 100%, 118.9 V vs. 129 V); these differences may stem from calibration and measurement resolution. “Experiment 3”. LVDAC-EMS measurements were observed to be proportional with increases in both voltage and current with duty cycle. DMM/oscilloscope measurements match for voltage but showed much lower current values compared to LVDAC-EMS. The discrepancy in current was due to a malfunctioning oscilloscope. Both experiments were consistent with theory learned in lectures.

## Conclusion

This lab properly demonstrated the usage of software-based (LVDAC-EMS) and hardware-based (DMM/oscilloscope) measurement systems for AC and DC circuits. Voltage and current properly changed due to altering probe or duty cycle values. Even though all experiments showed proper trends, an oscilloscope malfunction resulted in improper measurements for experiment 3. This highlights the importance of reliable and up-to-date technology for experiments in the electrical engineering field.

References

References

Toronto Metropolitan University. (n.d.). Lab-Volt Power Converter and Data Acquisition Platform [PDF]. D2L. https://courses.torontomu.ca/d2l/le/content/1064952/viewContent/6627400/View